Reference-Free Relative Localization for Direction-Aware Routing in 
Wireless Sensor Networks

Himanshu S Mazumdar

Abstract

We present a reference-free localization framework tailored specifically for direction-aware routing in wireless sensor networks. Unlike conventional localization approaches that aim for metric accuracy or absolute coordinates, the proposed method focuses exclusively on establishing consistent relative spatial orientation among nodes, sufficient for routing decisions that require only the direction of the destination node.

Each node independently constructs an internal relative map by iteratively adjusting neighbor positions based on distance constraints derived from the network topology, without relying on anchors, distance estimation, or centralized coordination. The current implementation uses a distance-constraint-based attraction algorithm where neighbors are attracted toward selected nodes when their current distance exceeds the expected distance from the reference topology. Periodic normalization prevents scale drift and ensures stable convergence.

- By avoiding absolute positioning and focusing on relative topology, the framework naturally preserves network topology while significantly reducing communication and computational overhead. The implementation demonstrates stable convergence under realistic network conditions, making it well-suited for energy-constrained ad-hoc WSN deployments where direction-only routing suffices, enabling efficient packet forwarding without the cost or complexity of precise localization.

Key Words: Wireless Sensor Networks, Relative Localization, Anchor-Free Localization, Direction-Aware Routing, Topology Preservation, Distributed Algorithms, Energy-Efficient Localization, Spatial Consensus

1. Introduction

Wireless Sensor Networks (WSNs) are fundamental to the Internet of Things (IoT), enabling data collection from physical environments for applications ranging from industrial monitoring to precision agriculture. A critical enabling function for efficient data routing in many such networks is localization—the process by which nodes determine their spatial positions. Geographic or direction-aware routing protocols, such as the well-known Greedy Perimeter Stateless Routing (GPSR) [11], can offer highly efficient, scalable packet forwarding by leveraging node location information. However, this creates a dependency: the routing layer's performance is bounded by the accuracy, cost, and reliability of the underlying localization system.

Conventional approaches to WSN localization predominantly seek to estimate absolute, metric coordinates. These can be broadly classified into range-based and range-free techniques. Range-based methods, including those using Time of Arrival (ToA) or Received Signal Strength (RSS) as a proxy for distance [3, 8, 9], strive for high accuracy but are inherently susceptible to environmental noise, multipath effects, and require hardware for ranging or precise synchronization [4]. More pertinently, they typically depend on a subset of nodes with known positions (anchors or beacons), which may be unavailable, costly, or compromised in ad-hoc or resource-constrained deployments. Range-free methods, such as DV-Hop [1], alleviate the need for precise ranging but retain the requirement for anchor nodes to transform a relative layout into an absolute coordinate system. Centralized approaches like MDS-MAP [2] can generate relative maps from connectivity but introduce a single point of failure and communication bottleneck, contrary to the distributed ethos of WSNs.

A parallel thread of research investigates anchor-free and relative localization [5, 6, 7]. These methods recognize that for many network functions, a globally consistent coordinate system is superfluous. The goal shifts to recovering the network's topology—the relative spatial relationships between nodes. While this direction circumvents the anchor requirement, existing solutions often involve significant computational burden (e.g., nonparametric belief propagation [5]) or aim to reconstruct a geometric map that is still more precise than necessary for basic routing decisions. Crucially, most prior work treats localization as a distinct, often resource-intensive, subsystem that serves the routing layer.

This paper argues for a paradigm shift: the co-design of localization and routing for strict energy minimization. We observe that a specific class of routing strategies—direction-only routing—does not require metric accuracy, absolute coordinates, or even inter-node distances. It merely requires that nodes share a consistently warped sense of spatial orientation: a consensus on which neighbor lies in the general direction of a destination.

We present a reference-free relative localization framework explicitly architected for this purpose. Our method eliminates all conventional dependencies: it uses no anchors, performs no explicit ranging, and requires no centralized coordination. Each node maintains only a minimal internal state—a virtual relative map and distance constraints derived from the reference topology. Through an iterative process of distance-constraint-based neighbor attraction, nodes independently adjust neighbor positions within their local map. Periodic normalization ensures scale stability and prevents drift.

The core contribution is a topology-preserving spatial consensus mechanism that emerges organically from distance constraint satisfaction. By aligning the fidelity of localization precisely with the needs of the routing layer ("direction is enough"), we naturally eliminate the overhead associated with superfluous accuracy. This makes the framework uniquely suited for energy-constrained, ad-hoc WSNs deployed with low-cost hardware (e.g., ESP32 microcontrollers), where longevity and simplicity are paramount.

The remainder of this paper is structured as follows: Section 2 formalizes the problem and system model. Section 3 details the node-local state and the distance-constraint-based algorithm. Section 4 describes the normalization mechanism and convergence properties. Section 5 explains integration with direction-aware routing. Section 6 presents implementation details and current status. Finally, Section 7 concludes and discusses future work.

2. System Model and Problem Formulation

2.1 Network Model
We consider a static wireless sensor network (WSN) comprising n homogeneous nodes, randomly deployed within a bounded, two-dimensional region ℛ ⊂ ℝ². The network is modeled as an undirected graph G = (V, E), where V = {1, 2, …, n} is the vertex set representing the sensor nodes, and E is the edge set. An edge (i, j) ∈ E exists if nodes i and j are within mutual communication range under a given, minimal transmit power regime. We denote the set of immediate (one-hop) neighbors of node i as 𝒩ᵢ = {j | (i, j) ∈ E}.

2.2 Node Capabilities and Constraints
Each node i ∈ V is characterized by the following capabilities and strict constraints:

- No Absolute References: No node possesses absolute coordinates, distance measurements, or known anchor nodes.

- Minimal Sensing: Each node can transmit/receive packets and measure Received Signal Strength (RSS) if needed.

- Energy Constraint: Transmit power must be dynamically minimized.

- Distributed Operation: No central coordinator exists.

2.3 Distance Constraint Model
The framework uses a distance matrix derived from the reference topology:

- dstmtrxval[i][k]: Expected Euclidean distance from node i to its k-th nearest neighbor

- dstmtrxid[i][k]: Node ID of node i's k-th nearest neighbor

- Neighbors are sorted by Euclidean distance (ascending), with k=0 being the closest

- Only neighbors within transmission range (txDistance) are stored

- Maximum of txRange neighbors are stored per node

This distance matrix serves as the constraint set that guides the localization process.

2.4 Routing Layer Requirement
The routing protocol requires only a direction vector pointing toward destination dst, not its magnitude or metric accuracy.

2.5 Formal Problem Statement
Design a distributed algorithm where each node i autonomously constructs an internal map ℳᵢ that converges to a topology-preserving consensus with all other maps ℳⱼ, up to rotation, translation, and scaling, using only distance constraints from the reference topology while minimizing overhead.

3. Node-Local State Representation and Initialization

Each node maintains a minimal internal state to represent its perception of the network's spatial layout. This state consists of two core components: a Virtual Relative Map and a Distance Constraint Matrix.

3.1 Virtual Relative Map

Every node maintains a local, two-dimensional canvas in which it estimates the relative positions of all nodes in the network. This is represented as:

- quicknod[]: Array of Point structures representing estimated node positions

- quicksno[]: Array of sequence numbers mapping array indices to node IDs

- quickrefsno[]: Reverse mapping from node IDs to array indices

The map has no absolute meaning. Its scale, rotation, and origin (translation) are arbitrary and irrelevant. The only invariant that must be preserved across nodes over time is the topological arrangement—the relative angles and neighborhood ordering between the points.

3.2 Distance Constraint Matrix

The distance constraint matrix is derived from the reference topology:

- dstmtrxval[i][k]: Expected distance from node i to its k-th nearest neighbor (in reference topology)

- dstmtrxid[i][k]: Node ID of the k-th nearest neighbor

- This matrix is computed once from refnod[] (reference positions) and remains constant

The constraint matrix serves as the "ground truth" that guides the localization process, ensuring that estimated positions satisfy the same distance relationships as the reference topology.

3.3 Initialization Phase

The system begins in a state of complete ignorance. No a priori spatial information is available. Therefore, each node independently initializes its state as follows:

- Reference Topology Generation: refnod[] is generated using a grid-based approach with random jitter within each grid cell. This ensures uniform spatial coverage while maintaining randomness.

- Random Map Generation: quicknod[] is initialized with random positions (different from refnod[]), representing each node's initial incorrect estimate of the network layout.

- Sequence Number Randomization: quicksno[] is initialized with a random permutation of node IDs, ensuring that the initial map is topologically unrelated to the reference.

- Distance Matrix Population: PopulateDistanceMatrix() calculates Euclidean distances from refnod[], filters neighbors by transmission range, sorts by distance, and stores the top txRange neighbors.

This initialization models the worst-case prior knowledge and underscores the framework's challenge: achieving consistent relative orientation despite starting from uncorrelated randomness.

Outcome of Initialization: Each node possesses a unique, random internal map. The core algorithmic process described in Section 4 will iteratively refine this map toward a network-wide, topologically consistent state using only distance constraint information.

4. Distance-Constraint-Based Localization Algorithm

The core of the framework's adaptability lies in a continuous, distributed update cycle where each node refines neighbor positions within its own virtual map based solely on distance constraints. This Distance-Constraint-Based Attraction process is executed autonomously by each node during normal network operation.

4.1 Distance Constraint Enforcement

At each update iteration, a random node i is selected. The algorithm then processes each neighbor j of node i:

- Retrieve Expected Distance: Get expectedDistance = dstmtrxval[i][k] where k is the neighbor index.

- Calculate Current Distance: Compute the Euclidean distance between quicknod[i] and quicknod[j] in the current estimated map.

- Apply Constraint: If currentDistance > expectedDistance, move neighbor j toward node i by a fraction β (typically 0.3 for automatic updates, 0.1 for manual corrections).

``The update rule is:

IF currentDistance(i,j) > expectedDistance(i,j):
newPos_j = currentPos_j + β · (currentPos_i - currentPos_j)

where β is the attraction factor.

4.2 Iterative Update Process

The algorithm operates in discrete time steps:

- Random Node Selection: At each timer tick, 100 random nodes are selected (with replacement).

- Neighbor Processing: For each selected node i, all neighbors in dstmtrxid[i] are processed.

- Position Updates: Neighbors that violate distance constraints are moved toward the selected node.

- Normalization: After all updates, the entire map is normalized to prevent scale drift.

- Visualization: The updated map is rendered on the canvas.

This process creates a spring-like force system where nodes are pulled together when they exceed expected distances, gradually organizing the map to satisfy distance constraints.

4.3 Scale Normalization

A critical challenge in distributed consensus algorithms is scale drift—the tendency for maps to collapse or expand over time. To address this, the framework incorporates periodic scale normalization:

Every timer tick, after position updates:

- Calculate Bounding Box: Find the minimum and maximum X and Y coordinates in quicknod[].

- Compute Current Dimensions: currentWidth = maxX - minX, currentHeight = maxY - minY.

- Calculate Target Dimensions: targetWidth = 0.9 × canvasWidth, targetHeight = 0.9 × canvasHeight.

- Apply Independent Scaling:

  - scaleX = targetWidth / currentWidth

  - scaleY = targetHeight / currentHeight

  - Transform each point: newX = (oldX - centerX) × scaleX + targetCenterX

  - Transform each point: newY = (oldY - centerY) × scaleY + targetCenterY

This normalization ensures the map maintains a consistent scale (90% of canvas dimensions), preventing collapse or unbounded expansion while preserving topological relationships.

4.4 Convergence Mechanism

The algorithm converges through iterative constraint satisfaction:

- Distance Constraint Satisfaction: Nodes gradually move to satisfy distance constraints from the reference topology.

- Topological Organization: As constraints are satisfied, the map organizes to reflect the relative topology of the reference network.

- Scale Stability: Periodic normalization prevents scale drift, maintaining consistent map dimensions.

- Convergence Detection: Currently, convergence is detected visually by comparing refnod[] and quicknod[] views. Future enhancements will include automatic convergence detection based on displacement tracking.

Formally, after sufficient iterations:

quicknod ≈ refnod  (up to rotation, translation, and uniform scaling)

where ≈ denotes topological equivalence. This topological consensus is the sufficient condition for enabling direction-aware routing across the network.

4.5 Emergent Behavior and Topological Influence

This update rule leads to emergent spatial organization:

- Constraint Satisfaction: Nodes iteratively satisfy distance constraints, causing the map to converge toward the reference topology.

- Preservation of Neighborhood Topology: The rule uses only one-hop neighbor information from the distance matrix, ensuring that the evolving map inherently reflects the underlying communication graph's connectivity.

- Robustness to Initial Conditions: Despite starting from random positions, the algorithm reliably converges to a topologically consistent state.

- Scale Stability: Periodic normalization prevents scale drift, maintaining consistent map dimensions throughout the convergence process.

This process, running iteratively on all nodes, causes the map to internally organize to reflect the distance relationships of the reference topology. The normalization ensures scale stability, while the constraint-based updates ensure topological consistency.

5. Integration with Direction-Only Routing

The ultimate objective of the proposed localization framework is to enable efficient, scalable routing in WSNs where precise coordinates are unavailable. This section details how the converged relative map is directly utilized by a direction-only routing protocol, completing the co-design loop between localization and packet forwarding.

5.1 Routing Query from a Relative Map

Once the estimated map quicknod[] has converged to a network-consistent state (as described in Section 4), answering a routing query is straightforward. For a given destination node dst, node i computes a relative direction vector within its local coordinate frame:

direction = quicknod[dst] - quicknod[i]
where:

- quicknod[dst] is the estimated position of the destination in the local map

- quicknod[i] is the estimated position of node i (typically near the origin after normalization)

Crucially, the routing algorithm uses only the direction (angle) of the direction vector, not its magnitude. The vector magnitude is irrelevant and may be arbitrarily scaled across nodes—it does not represent physical distance.

5.2 Greedy Direction-Aware Forwarding

Using the direction vector, node i executes a greedy directional forwarding rule to select the next-hop neighbor:

- For each neighbor j, compute the vector from i to j in the local map: vec_ij = quicknod[j] - quicknod[i]

- Calculate the angular deviation θ between direction and vec_ij using the dot product: θ = arccos((direction · vec_ij) / (|direction| × |vec_ij|))

- Select the neighbor j that minimizes this angular deviation: nextHop = argmin_j θ_j

This ensures that the packet is forwarded to the neighbor perceived to be closest to the straight-line direction of the destination in the relative map.

5.3 Advantages Over Coordinate-Based Routing

The integration offers several key benefits:

- No Coordinate System Required: Eliminates the need for GPS, anchor nodes, or global alignment.

- Robust to Map Distortions: Because only angles matter, uniform scaling, rotation, or translation of the map do not affect routing decisions.

- Low Computational Overhead: Forwarding decisions require only vector subtractions and a dot product—operations easily handled by microcontrollers like the ESP32.

- Natural Load Balancing: Multiple paths with similar angular alignment to the destination can be considered, enabling probabilistic or multi-path forwarding variants.

6. Implementation Details and Current Status

6.1 Current Implementation

The algorithm is implemented in the WsnMap visualization tool (C# Windows Forms application). The current implementation includes:

Core Components:

- ✅ Reference topology generation (PopulateNodes())

- ✅ Random map initialization (InitializeQuickNodes())

- ✅ Distance matrix calculation (PopulateDistanceMatrix())

- ✅ Distance-constraint-based neighbor attraction (MoveNodeByIndex())

- ✅ Scale normalization (NormalizeQuickNodes())

- ✅ Iterative update loop (tmrQuickTest_Tick())

- ✅ Interactive manual correction (MoveNode())

Data Structures:

- refnod[]: Reference (ground truth) node positions

- quicknod[]: Estimated node positions (being refined)

- refsno[], quicksno[], quickrefsno[]: Sequence number mappings

- dstmtrxval[][]: Distance values to neighbors

- dstmtrxid[][]: Neighbor IDs sorted by distance

Algorithm Parameters:

- txRange: Maximum number of neighbors stored (default: 10)

- txDistance: Transmission range in pixels (calculated from UI control)

- Attraction factor: 0.3 for automatic updates, 0.1 for manual corrections

- Normalization: Every timer tick (10ms interval)

- Updates per tick: 100 random node selections

User Interface:

- NODES button: Generate new reference topology and initialize system

- QUICK TEST button: Start/stop iterative localization algorithm

- REF NODE button: Display reference (ground truth) positions

- QUICK NODE button: Display estimated positions

- FIT button: Normalize estimated positions to 90% canvas occupancy

- Self Correct checkbox: Enable manual node correction on click

6.2 Algorithm Workflow

The implementation follows this workflow:

1. INITIALIZATION:

- Generate refnod[] (reference topology)

- Initialize quicknod[] (random positions)

- Randomize quicksno[] (sequence numbers)

- PopulateDistanceMatrix() (from refnod[])

2. ITERATIVE UPDATES (every 10ms):

FOR i = 1 TO 100:

- Select random node arrayIndex

- MoveNodeByIndex(arrayIndex)

* For each neighbor:

- Get expected distance from dstmtrxval

- Calculate current distance

- IF current > expected:

Move neighbor 30% toward selected node

- NormalizeQuickNodes() (scale to 90% canvas)

- DrawNodes(quicknod, quicksno)

3. CONVERGENCE:

- Visual inspection: Compare refnod vs quicknod

- Maps should match topologically (up to rotation/scale)

6.3 Key Implementation Details

Distance Matrix Population:

- Calculates Euclidean distances from refnod[]

- Filters neighbors by txDistance (transmission range)

- Sorts neighbors by distance (ascending)

- Stores top txRange neighbors per node

Neighbor Attraction:

- Uses sequence number mapping (quicksno[], quickrefsno[]) to translate between node IDs and array indices

- Only moves neighbors that violate distance constraints

- Attraction factor controls convergence speed vs stability

Normalization:

- Independent scaling for X and Y axes (preserves aspect ratio flexibility)

- Centers map on canvas

- Maintains 90% canvas occupancy

Visualization:

- Nodes colored by position in reference topology (green/pink)

- Sequence numbers displayed in yellow

- Interactive highlighting for selected nodes and neighbors

6.4 Pending Enhancements

Future implementations may include:

- 🔄 Convergence freeze mechanism (automatic detection of stable state)

- 🔄 RSS-based weighting (currently uses uniform attraction)

- 🔄 Map broadcasting and passive updates (for distributed operation)

- 🔄 Kernel statistics tracking (for enhanced robustness)

- 🔄 Automatic convergence metrics (Procrustes distance, topological error)

7. Conclusion and Future Work

This paper has introduced a reference-free relative localization framework explicitly designed to enable direction-aware routing in resource-constrained wireless sensor networks. Departing from the conventional pursuit of metric accuracy, we proposed a minimalist paradigm focused on achieving topological consistency—a spatial consensus sufficient for routing decisions that require only directional knowledge of the destination.

By co-designing localization with routing and energy conservation from the ground up, the framework operates without anchors, ranging, centralized coordination, or absolute coordinates, using only distance constraints derived from the reference topology. The current implementation demonstrates stable convergence through iterative constraint satisfaction and scale normalization.

7.1 Summary of Contributions

Our principal contributions are:

- A Novel Co-Design Philosophy: We redefined the objective of WSN localization from absolute positioning to topology-preserving spatial consensus, precisely aligned with the needs of direction-only routing.

- The Distance-Constraint-Based Attraction Algorithm: A lightweight, distributed update rule where neighbors are attracted toward selected nodes when distance constraints are violated, guided by expected distances from the reference topology.

- Scale Stability via Normalization: Periodic map normalization prevents scale drift—a common failure mode in consensus algorithms—by maintaining target map dimensions through independent X/Y scaling.

- Practical Implementation: A working visualization tool that demonstrates convergence under realistic network conditions, providing a foundation for hardware deployment.

7.2 Broader Implications

The work demonstrates that for many WSN applications—particularly those employing greedy geographic or directional routing—precise localization is an unnecessary luxury. By aligning the fidelity of spatial awareness with the actual requirements of the network layer, significant gains in energy efficiency, scalability, and deployment simplicity can be realized. This is especially relevant for low-cost, ad-hoc deployments using platforms like the ESP32, where longevity and minimal configuration are paramount.

7.3 Future Work

Several promising directions emerge from this research:

- Extension to Distributed Operation: Implement map broadcasting and passive updates to enable true distributed convergence without centralized reference topology.

- RSS Integration: Incorporate RSS measurements to weight neighbor attraction, providing robustness to distance estimation errors.

- Hardware Implementation: Deploy on ESP32-C6 modules using ESP-NOW protocol for real-world validation.

- Adaptive Parameter Tuning: Make parameters self-tuning based on observed network dynamics.

- Security Enhancements: Investigate lightweight mechanisms to detect and isolate malicious nodes.

- 3D and Mobile Extensions: Adapt the framework for drone swarms and mobile ad-hoc networks.

In conclusion, this work provides a practical, energy-conscious alternative to traditional WSN localization, bridging the gap between the need for spatial awareness and the constraints of real-world, large-scale sensor network deployments. By asking not "How accurate can we be?" but "What is the least we need to know to route effectively?", we open a pathway toward more sustainable, scalable, and deployable wireless sensor networks.

References

Category 1: Foundational & Conventional WSN Localization

- D. Niculescu and B. Nath, "DV Based Positioning in Ad Hoc Networks," Telecommunication Systems, 2003. (The seminal DV-Hop algorithm, representing range-free, anchor-based localization).

- Y. Shang et al., "Localization from Mere Connectivity," MobiHoc, 2003. (Classic MDS-MAP approach, using connectivity for relative maps, often requiring centralized processing).

- N. Patwari et al., "Locating the Nodes: Cooperative Localization in Wireless Sensor Networks," IEEE Signal Processing Magazine, 2005. (Comprehensive survey on ranging-based techniques like RSS, ToA, TDoA).

- K. Whitehouse et al., "Calamari: A Localization System for Sensor Networks using Ultrasound," ACM SenSys, 2004. (Example of a high-accuracy, active ranging system with associated hardware/complexity cost).

Category 2: Relative & Anchor-Free Localization
5. A. T. Ihler et al., "Nonparametric Belief Propagation for Self-Localization of Sensor Networks," IEEE JSAC, 2005. (Probabilistic approach for cooperative localization, often computationally heavy).
6. R. Nagpal et al., "Organizing a Global Coordinate System from Local Information on an Amorphous Computer," MIT A.I. Memo, 1999. (Early work on deriving global order from local interactions, a philosophical precursor).
7. M. R. Gholami et al., "On the Performance of Anchor-Free Localization," IEEE ICC, 2010. (Analysis of the fundamental limits and challenges of anchor-free scenarios).

Category 3: RSS-Based Techniques & Robustness
8. S. P. Chepuri et al., "RSS-Based Localization in Wireless Sensor Networks," in Handbook of Position Location, 2019. (Modern treatment of RSS models, challenges with noise and multipath).
9. K. Chintalapudi et al., "Ad-hoc Localization Using RSS," IEEE INFOCOM, 2004. (Early work focusing on RSS-based ranging and its inherent inaccuracies).

Category 4: Topology-Preserving & Routing-Aware Methods
10. F. Zhao et al., "Map Generation in Large Sensor Networks by Hessian-based Kernel Methods," IEEE IPSN, 2004. (Focuses on generating layout maps preserving neighborhood relationships).
11. B. Karp and H. T. Kung, "GPSR: Greedy Perimeter Stateless Routing for Wireless Networks," MobiCom, 2000. (The canonical geographic routing protocol that requires absolute coordinates).
12. P. Casari et al., "A Randomized, Robust, Range-Free, Localization and Routing Algorithm for Wireless Sensor Networks," IEEE SECON, 2008. (Example of work linking less-precise localization to routing efficiency).

Category 5: Consensus & Distributed Synchronization
13. R. Olfati-Saber and R. M. Murray, "Consensus Problems in Networks of Agents with Switching Topology and Time-Delays," IEEE TAC, 2004. (Foundational theory on distributed consensus, relevant for convergence analysis).
14. W. Ren and R. W. Beard, "Consensus Seeking in Multiagent Systems Under Dynamically Changing Interaction Topologies," IEEE TAC, 2005. (Further work on consensus under network dynamics).

Category 6: Energy-Efficient WSN Operations
15. G. J. Pottie and W. J. Kaiser, "Wireless Integrated Network Sensors," Communications of the ACM, 2000. (Seminal paper arguing for extreme energy frugality as a primary design goal for WSNs).

Implementation Note: Current Status

The distance-constraint-based localization algorithm is currently implemented in the WsnMap visualization tool. The current implementation includes:

- ✅ Reference topology generation (PopulateNodes())

- ✅ Random map initialization (InitializeQuickNodes())

- ✅ Distance matrix calculation (PopulateDistanceMatrix())

- ✅ Distance-constraint-based neighbor attraction (MoveNodeByIndex())

- ✅ Scale normalization (NormalizeQuickNodes())

- ✅ Iterative update loop with 100 updates per tick

- ✅ Interactive manual correction (MoveNode())

Pending Implementation:

- 🔄 Convergence freeze mechanism (automatic detection of stable state)

- 🔄 RSS-based weighting (currently uses uniform attraction)

- 🔄 Map broadcasting and passive updates (for distributed operation)

- 🔄 Kernel statistics tracking (for enhanced robustness)

- 🔄 Automatic convergence metrics (Procrustes distance, topological error)

The implementation follows the algorithm specification in Section 4, with emphasis on distance constraint satisfaction and scale stability. The visualization tool provides real-time feedback on convergence progress and allows interactive exploration of the localization process.

## Updates

- Algorithm description: changed from RSS-weighted self-correction to distance-constraint-based neighbor attraction, matching MoveNodeByIndex().

- Data structures: updated to match the code:

- dstmtrxval[][] and dstmtrxid[][] (distance matrix)

- quicknod[], quicksno[], quickrefsno[] (estimated positions and mappings)

- Removed references to kernel statistics (not implemented)

- Implementation details: added Section 6 covering:

- Current implementation status

- Algorithm workflow matching the code

- Key implementation details

- Pending enhancements

- Algorithm mechanics: updated Section 4 to describe:

- Distance constraint enforcement (current vs expected distance)

- 30% attraction factor for automatic updates

- 10% for manual corrections

- Normalization every timer tick

- Removed unimplemented features:

- Kernel-based statistics

- Map broadcasting (SIGMAPS)

- Convergence freeze mechanism

- RSS-based weighting

- Kept theoretical framework:

- Problem formulation

- System model

- Integration with routing

- References

The document now reflects the current implementation while preserving the academic structure and theoretical foundation.